Project overview

This repository implements a single cycle, 32-bit ARM-like CPU in Verilog/SystemVerilog. The design separates controller and datapath, externalizes instruction and data memories, supports a small but representative ARM instruction subset (ADD, SUB, AND, ORR, LDR, STR, B), and implements full ARM condition codes with pipelined flag registers. The following sections explain every module, signal path, and the exact stepwise process an instruction goes through from fetch to writeback. Use this as a definitive reference for documentation, debugging, simulation, and extension.

Module by module detailed explanation

top.v

- Role: top-level system wiring. Instantiates CPU core, instruction memory, and data memory. Exposes a simple SoC-like interconnect: PC drives imem; ALUResult and WriteData connect to dmem; dmem returns ReadData to CPU.

- Important signals: PC (from cpu), Instr (from imem), DataAdr and WriteData (to dmem), ReadData (from dmem), MemWrite (write enable to dmem).

- Note: dmem uses word addressing via a[31:2] so addresses must be word aligned.

cpu.v

- Role: composes controller and datapath. Controller produces control signals based on instruction bits and flags; datapath implements PC, registers, ALU, and memory interface.

- Interface summary:

  - Inputs: clk, reset, Instr (32 bits), ReadData (from dmem)

  - Outputs: PC (to imem), MemWrite, ALUResult, WriteData (to dmem)

- Dataflow: controller ← Instr[31:12], ALUFlags ← datapath, controller → control signals → datapath.

imem.v

- Role: simple instruction ROM. Loads memfile.dat with $readmemh into a RAM array and outputs the instruction at index PC[31:2].

- Important: word indexing; memfile.dat must contain 32-bit words in hex.

dmem.v

- Role: synchronous word RAM. Combinational read via RAM[address[31:2]]; synchronous write at posedge clk when we asserted.

- Caveat: reads are immediate, writes occur on posedge. If a load and a store target same address in same cycle, simulator semantics determine the observed value.

datapath.v

- Contains:

  - PC logic: PC register (flopr), two adders to compute PC+4 and PC+8, PC mux selecting between PC+4 and Result (branch target).

  - Register file interface and read address selection via RegSrc multiplexers.

  - Register file instantiation: regfile supports 15 RF entries (rf[0..14]) with read ports and a special-case R15 supplied externally as r15 input (PCPlus8).

  - Extender: immediate generation for multiple ImmSrc encodings.

  - ALU input mux: ALUSrc selects between immediate and register-sourced data.

  - ALU instantiation returns ALUResult and ALUFlags.

  - Writeback mux: MemtoReg selects between ALUResult and memory ReadData for register writeback.

- Control inputs: RegSrc[1:0], RegWrite, ImmSrc[1:0], ALUSrc, ALUControl[1:0], MemtoReg, PCSrc.

- Outputs: ALUFlags[3:0], PC, ALUResult, WriteData.

regfile.v

- Structure: rf[14:0] provides 15 general registers. Reads return r15 externally when ra == 4'b1111. Writes occur on posedge clk when we3 asserted; writing to address 1111 is not mapped into the rf array (special handling must be explicit elsewhere).

- Important: This design treats register index 15 as a special read-only input r15; writes to R15 require explicit logic in the datapath or controller to write PCPlus8 into rf[] instead of writing index 15.

alu.v

- Functionality:

  - Implements 32-bit AND, OR, ADD, SUB selected by ALUControl: 2'b10 => AND, 2'b11 => OR, 2'b00 => ADD, 2'b01 => SUB.

  - Produces ALUResult and flags: Negative = ALUResult[31], Zero = (ALUResult == 0), Carry captured via concatenation {Carry, ALUResult} = arithmetic result, Overflow computed with standard signed overflow formulas.

- Edge behaviors:

  - Carry from subtraction requires careful interpretation: using {Carry, ALUResult} = SrcA - SrcB yields a result whose top bit is the carry/borrow; verify ARM semantics if matching reference precisely.

extend.v

- Handles three immediate formats:

  - ImmSrc 00: zero-extend Instr[7:0]

  - ImmSrc 01: zero-extend Instr[11:0]

  - ImmSrc 10: sign-extend Instr[23:0] then left shift 2 bits for branch displacement

- Checkpoint: branch immediate uses {{6{Instr[23]}}, Instr[23:0], 2'b00} to generate signed, shifted 32-bit branch offset.

decoder.v

- Main decode:

  - Produces control vector controls = {Branch, MemtoReg, MemW, ALUSrc, ImmSrc, RegW, RegSrc, ALUOp}.

  - Uses casex on Op and Funct to choose instruction class and set controls.

  - Defaults to x for unknown opcodes—consider replacing with safe defaults and diagnostics.

- ALU decode:

  - When ALUOp asserted, selects ALUControl based on Funct bits and sets FlagW bits driven by S bit for flag updates.

- PCS output:

  - PCS asserted when Rd == 4'b1111 and RegW true, or when Branch true. This signals write to PC or branch target update.

condlogic.v and condcheck.v

- Purpose: Implement ARM conditional execution.

- condcheck maps 4-bit Cond into logic that tests stored Flags [N Z C V] and generates CondEx (true if condition met).

- condlogic:

  - Uses two flopenr instances to latch flag halves (high bits and low bits) under enable flags.

  - FlagWrite = FlagW & {2{CondEx}} ensures flags update only if the instruction’s condition is true.

  - RegWrite, MemWrite, PCSrc are gated by CondEx so instructions only take effect when condition satisfied.

flopr.v and flopenr.v

- flopr: synchronous register with asynchronous reset used for PC.

- flopenr: parameterizable width register with enable and asynchronous reset used to implement flag pipeline with write enables.

testbench.sv and aluTest.v

- testbench.sv instantiates top, toggles reset, runs clock, and at simulation end checks register contents to report pass/fail; produces a VCD file for waveform inspection.

- aluTest.v is a focused ALU unit test harness.

Step by step process for a single instruction

This describes exactly what happens during one cycle for any instruction — fetch, decode, operand read, execute, memory, writeback, flag update, and PC update. Follow these steps to trace any instruction in simulation or to reason about correctness.

- Instruction Fetch (combinational, prior to rising clock)

  - PC holds value from previous cycle (registered by flopr).

  - imem outputs Instr = RAM[PC[31:2]]. Instr is immediately available to controller and datapath combinationally.

  - datapath combinationally computes PCPlus4 = PC + 4 and PCPlus8 = PCPlus4 + 4.

- Decode and Control Generation (combinational)

  - Controller slices Instr[31:12] and passes it to decoder.

  - Decoder casex on Op, Funct and Rd to generate the 10-bit control vector controls and determine ALUOp.

  - If ALUOp asserted, ALU decode sets ALUControl and FlagW bits according to Funct and S bit.

  - Controller passes FlagW and ALUFlags to condlogic for conditional evaluation.

- Condition Evaluation and Write Enables (combinational)

  - condcheck evaluates the instruction Cond field against stored Flags from the previous cycle and sets CondEx.

  - FlagWrite = FlagW & {2{CondEx}} such that flags will be updated on this instruction only if condition true.

  - RegWrite = RegW & CondEx, MemWrite = MemW & CondEx, PCSrc = PCS & CondEx. These control whether state changes happen on the upcoming posedge clock.

- Operand Selection and Immediate Generation (combinational)

  - RA1 and RA2 are chosen via RegSrc multiplexers from instruction fields or R15 constant.

  - regfile outputs rd1 and rd2 based on RA1 and RA2; if RA == 1111, regfile returns external r15 value (PCPlus8).

  - ExtImm computed by extend.v according to ImmSrc.

  - srcbmux chooses SrcB = ExtImm if ALUSrc asserted; otherwise SrcB = WriteData or register read as wired.

- ALU Execution and Flag Computation (combinational)

  - ALU receives SrcA, SrcB, ALUControl and computes ALUResult.

  - ALU computes Carry via concatenation with result and computes Overflow using signed overflow boolean formulas.

  - Negative = ALUResult[31]; Zero = (ALUResult == 0).

  - ALUFlag vector {N,Z,C,V} is presented to condlogic for possible writing.

- Memory interface and result selection (combinational)

  - DataAdr = ALUResult.

  - dmem read port supplies ReadData = RAM[DataAdr[31:2]] immediately.

  - resmux selects Result = ReadData if MemtoReg asserted, else ALUResult.

  - WriteData for stores is sourced from regfile (forwarded via datapath wiring).

- Synchronous update at rising clock

  - On posedge clk (if reset not asserted):

    - If RegWrite true then regfile writes wd3 = Result into rf[wa3 = Instr[15:12]].

      - If wa3 == 1111 and design intends link semantics, special handling needed to write PCPlus8 into a real register slot; otherwise writing index 1111 will be ignored by regfile implementation.

    - flopenr flag registers update their stored flags if respective enables from FlagWrite are true.

    - dmem performs synchronous write if we asserted: RAM[DataAdr[31:2]] <= WriteData.

    - PC register flopr updates PC <= PCNext where PCNext is either PCPlus4 or Result depending on PCSrc that was gated by CondEx.

- After clock edge

  - New PC is used by imem in the next cycle.

  - New register values and flags are visible to the next cycle’s decode and condition evaluation.

  - Memory writes from this cycle are available for immediate subsequent reads of dmem (depending on simulator memory semantics).

Key implementation details, assumptions, and pitfalls you must know

- Register file sizing and R15 semantics

  - rf[14:0] contains 15 entries; ra==1111 returns external r15 input (PCPlus8) rather than rf[15]. There is no implicit write to rf[15] in regfile; writing to R15 must be implemented explicitly via writeback routing when needed.

- Memory alignment

  - dmem uses RAM[a[31:2]] so accesses are word aligned. Byte or halfword accesses are not supported.

- Flag timing

  - ALU flags produced by the ALU are only written to flag registers on the posedge clk when FlagWrite enables and CondEx are true. Flags updated by instruction I become visible to conditional checks for instruction I+1.

- Carry semantics on subtraction

  - Using {Carry, ALUResult} = SrcA - SrcB yields a carry representation that may differ from the ARM standard borrow interpretation without a further inversion step; verify against ARM references if exact ISA parity is required.

- Default control values

  - decoder uses 10'bx for default. In simulation, unknown control bits propagate X states. Assign safe defaults or add explicit illegal-op handling to aid debugging.

- Reset behavior

  - flopr and flopenr use asynchronous reset (posedge reset in sensitivity). Ensure testbench applies reset long enough for proper initialization.

- Combinational loops and X propagation

  - Avoid relying on simulator-specific resolution for undefined control vectors. When adding new instruction encodings, initialize controls properly.

- Tooling

  - Some files use SystemVerilog style keywords like logic. Use a simulator/flags that accept -g2012 or full SystemVerilog support.

Step by step simulation and debugging process

Follow this reproducible process for simulation, waveform inspection, and debugging.

- Prepare simulator environment

  - Recommended: Icarus Verilog (iverilog) for quick iteration. Use ModelSim or Questa for richer debug.

  - Compile with SystemVerilog support: iverilog -g2012 -o simv *.v testbench.sv

- Run simulation

  - Execute: vvp simv

  - Testbench produces VCD file named by $dumpfile (extender.vcd or as coded). It also prints pass/fail messages.

- Inspect waveforms

  - Open VCD in GTKWave: gtkwave extender.vcd

  - Typical signals to view: clk, reset, PC, Instr, PCPlus4, PCPlus8, ALUResult, ALUFlags[N Z C V], RegWrite, MemWrite, ReadData, WriteData, regfile contents (dut.cpu.dp.rf.rf[]), flopenr flag registers.

- Unit test focused modules

  - ALU: run aluTest.v, check N/Z/C/V on boundary cases.

  - Extend: add a tiny test to exercise ImmSrc values and ensure sign extension and left shift for branches are correct.

  - Decoder: feed Op/Funct combos and assert control vector matches expected bit patterns.

- Add runtime diagnostics

  - Temporarily add $display statements in decoder and condlogic to print per-instruction information: time, PC, Instr, cond, control bits, ALUControl, FlagWrite, CondEx, RegWrite, MemWrite.

  - Use $strobe to ensure you capture post-clock update values if needed.

- Common failure modes and how to identify them

  - X propagation through controls: look for X in control signals in the waveform; fix decoder defaults.

  - Wrong branch behavior: inspect PCS, PCSrc, and Result mux inputs, check immediate sign/shift in extend.v.

  - Incorrect flag behavior: use ALU unit tests to ensure Carry and Overflow computed as expected; check flopenr enable lines and CondEx gating.

  - Register write semantics for R15: if R15 writebacks are missing or incorrect, verify how writeback selects Result and whether wa3==1111 is handled.

- Make tests deterministic

  - Use fixed reset duration: drive reset high for several cycles before releasing, then begin testing.

  - Add automated checks in testbench that compare expected register/memory values at defined simulation time and $display pass/fail.

Ready checklist and recommended README items to include

- Quick commands

  - Compile: iverilog -g2012 -o simv *.v testbench.sv

  - Run: vvp simv

  - Waveform: gtkwave extender.vcd

- What to document in README immediately

  - Module responsibilities table mapping filenames to roles.

  - Instruction field mapping and the bit ranges used by decoder and extender.

  - Register file layout and R15 special-case explanation.

  - Memory model: word addressing, alignment requirement, synchronous writes.

  - Flags semantics formulas for C and V for both ADD and SUB, and explanation of when flags update.

  - Known limitations: single cycle design, no byte/halfword support, explicit R15 write semantics required.

- Suggested small additions to repo

  - Add a small assembler-to-memfile script to translate readable assembly into memfile.dat..

  - Provide unit test harnesses for extend.v, regfile.v, decoder.v, and ALU corner cases.

  - Replace decoder default x assignments with safe defaults and an $display or $fatal for illegal opcodes.